# Contact Wire Irregularity Stochastics and Effect on High-speed Railway Pantograph-Catenary Interactions

Yang Song, Member, IEEE, Zhigang Liu, Senior Member, IEEE, Anders Rønnquist, Petter Nåvik, Zhendong Liu

Fig. 1. Schematic of a pantograph-catenary system

Fig. 2. Local dropper defect

Abstract—In high-speed rail operations, contact wire irregularity in a catenary is a common disturbance to the pantograph-catenary interaction performance, which directly affects the quality of current collection. To describe the pointwise stochastics of contact wire irregularity, the power spectral density (PSD) function for contact wire irregularity is proposed, and its effect on the pantograph-catenary interaction is investigated. First, a preprocessing procedure is proposed to eliminate the redundant information in the measured irregularities based on the ensemble empirical mode decomposition (EEMD). Then, the upper envelope of the irregularity, which contains all the information regarding the dropper positions on the contact wire, is extracted. A form of the PSD function is suggested for contact wire irregularities. A methodology is proposed to include the effect of random irregularities in the assessment of the interaction performance of a pantograph-catenary. A developed target configuration under dead load (TCUD) method is employed to calculate the initial configuration of the catenary, in which the dropper points on the contact wire are placed on their exact positions. Finally, the effect of the random contact wire irregularities on the contact force is investigated through 500 numerical simulations at each operating speed. The present results indicate that random irregularities have a direct impact on the pantograph-catenary contact, including an increment in the dispersion of the contact force statistics. The stochastic analysis shows that over 70% of the results with irregularities are worse than the ideal result without irregularities.

Index Terms—High-speed railway, Current collection quality, Contact wire irregularity, PSD, Contact force, Pantograph-catenary interaction, Probability analysis

# INTRODUCTION

I

N modern high-speed railway systems, the catenary constructed along the railroad interacts with the pantograph mounted on the carbody roof to transmit electric current to the engine inside the train, as shown in Fig. 1. The contact between the pantograph and catenary is the only source of power for the high-speed train [1]. The sliding contact performance of the pantograph collector and the contact wire is of critical importance to the quality of current collection, which not only restricts the maximum speed but also affects the safe and stable operation of high-speed railways. However, the pantograph-catenary system is affected by various disturbances from both the environment and the system itself. Normally, the external disturbances include wind load [2], locomotive vibration [3], temperature variance [4], ice layers [5] and electromagnetic interference [6], while the disturbances from the system itself include contact wire wear [7], defects in components [8] and contact wire irregularity (CWI) [9]. These disturbances are the major causes of deterioration in the quality of current collection.

Many studies have been performed to reduce the negative effects of these disturbances on the pantograph-catenary interaction. For the wind load, a stochastic wind field is constructed along the catenary to evaluate the nonlinear behaviour of wind-induced vibration and investigate the sensitivity of the structural parameters to the wind-resistance capability [10]. Considering the galloping of the catenary in extreme conditions, the aerodynamic instability [11] of the contact wire is investigated using computational fluid dynamics. For vehicle-track excitation, a vehicle-track-pantograph-catenary model is constructed [12], and the effect of random rail irregularities on the pantograph-catenary interaction is analysed [13]. The temperature variance caused by pantograph-catenary arcing is monitored by optical fibre sensors [14], and the temperature distribution is analysed during the pantograph lowering process [15]. The arcing caused by an ice layer covering the contact wire is considered in [10], and its effect on the electrical transmission quality is studied [11]. The wear on the contact interface is inspected by computer vision [17], and the prediction model of the wear on the pantograph strip is proposed based on the experimental results [18]. Using historical data from the Dutch railway network, the evolution of contact wire wear is investigated [19]. The development of the modern computer vision method provides more solutions for the identification of defects in catenary components, such as the distortion of the stagger value [20], fasteners on the support device [21], bird nests in the catenary [22] and cracks in insulators [23]. Considering the defects in the catenary, some methodologies are proposed to establish the catenary model with tension loss [24] and local sag defects [25].

Apart from the abovementioned disturbances, another common fault in the early service stage of the catenary is CWI. Long-term service, mounting errors, contact incidents, and poor maintenance may result in plastic deformation, loosening and even breakage of droppers, which represents the major source of variation in the contact wire height, as illustrated in Fig. 2, and directly affects the vertical contact quality of the pantograph and the catenary. The significant deviation of the real contact wire height with respect to its design position has been acknowledged in [26]. However, in most previous studies, only the design data are used to model the catenary. CWI is first involved in the study of pantograph-catenary interaction in [27], in which CWI is taken as the additional displacement of the contact wire in the calculation of the contact force. In [28], the important implication of the contact wire pre-sag on the interaction performance of the pantograph-catenary is indicated through numerical simulations. An analytical model to evaluate the contribution of the catenary geometry to the contact force fluctuation is developed in [29]. In [30], a time-frequency representative is presented to locate and identify the CWI from the contact force based on the Zhao–Atlas–Marks time-frequency distribution (ZAMD). Based on this work, the concept of the structure wavelength of the catenary is proposed to quantify the negative effect of CWI [31]. As the detection technique develops, field data are collected to study the impact of contact wire irregularities on the pantograph-catenary interaction [32]. However, there are still some significant shortcomings in the existing works on this research topic. First, most works treat CWI as additional displacement in the calculation of the contact force. This assumption is unrealistic, as CWI is normally caused by plastic deformation of the catenary geometry, which should be described in the initial configuration of the catenary. Second, CWI is a random disturbance to the pantograph-catenary system. The stochastic representation of CWI should be discussed. In the research field of track irregularities, the power spectral density (PSD) function is normally used to represent the frequency distribution of a random time series. Different standard PSD forms of track irregularities have been proposed for networks in different countries, such as China [33], the United States [34] and France [35]. Normally, the standard PSD of track irregularities can be used for two objectives. One objective is to generate the random history of track irregularities for the simulation of vehicle-track interaction, and the other is to set up a benchmark for the assessment of the health condition of the track.

This paper is the first to attempt to perform a stochastic analysis of the pantograph-catenary interaction with random CWI. The measured CWI is collected from the China high-speed network, and its frequency distribution is analysed by power spectrum estimation. Through EEMD (ensemble empirical mode decomposition), the CWI signal is reconstructed by eliminating the measurement errors and redundant information. Then, the CWI upper envelope (CUE) is adopted to describe the positions of the dropper points on the contact wire. The proper form of the CUE PSD is proposed, and the algorithm for generating the history from the PSD is discussed. Using the vertical positions of the dropper points on the contact wire, a nonlinear finite element approach is proposed to construct the catenary. The TCUD (target configuration under dead load) method is used to precisely solve the initial configuration of the catenary with the random CWI. Using the proposed model, the contact forces at different speeds are evaluated. Probabilistic analysis is adopted to investigate the effect of random CWI on the contact force.

# Analysis of measured CWI

The measured CWI data used in this work are collected from the China high-speed network via an inspection vehicle that runs regularly to monitor the static parameters of the catenary. The nominal height of the contact wire is removed from the measured data, and the corresponding CWI is presented in Fig. 3. The global view in Fig. 3 (a) shows that there are some extremely large peaks in the measurement data. Most of the peaks are caused by measurement errors. Some of the peaks represent overlap, which is a transition between two normal tensile sections of the catenary. Normally, the contact wire is higher at an overlap than in a normal section, but the pantograph runs at the same height, and the overlap does not cause large variations in the contact wire height. The local view in Fig. 3. (b) shows the CWI from 2260 m to 2330 m. The CWI is mainly determined by the dropper positions on the contact wire. The contact wire sag between two adjacent droppers is purely caused by gravity. However, some high-frequency measurement errors are significantly observed. The PSD analysis of this portion of CWI is presented in Fig. 4. The PSD peaks appear at frequencies relevant to the span length, half-span length, double dropper interval and dropper interval. The

Fig. 5. IMFs extracted from measured CWI: (a-o) denote IMFs 1-15; (p) denotes residual

frequency components of CWI higher than the dropper interval frequency are seemingly relevant to wear, measurement errors, and sag caused by gravity. However, the

Fig. 4. PSD of measured CWI

detection of contact wire wear is out of the accuracy range of the measurement equipment, according to its specifications. The sag caused by gravity is deterministic by the mechanical rules. The frequency components smaller than the span length are

Fig. 3. Measured CWI: (a) Global view; (b) Local view

mostly relevant to the overlap sections. Therefore, the CWI is mainly determined by the vertical positions of dropper points on the contact wire. In the next section, a preprocessing procedure is proposed to eliminate the redundant information in the measured CWI based on EEMD. Then, the upper envelope of the measured CWI is extracted to represent the positions of the droppers on the contact wire. Finally, a standardised formulation is presented to fit the PSD of the CWI upper envelope (CUE).

# Construction of PSD for CWI

Based on the idea in the previous section, the standardised PSD for CWI is presented in this section. A three-step procedure is proposed to construct the PSD function from the measured CWI. The first step is preprocessing with EEMD. The second step is extraction of the upper envelope, and the last step is curve fitting of the CUE PSD. Furthermore, the algorithm to generate the history of the dropper positions from the PSD is also discussed.

## Preprocessing with EEMD

EMD is a data-driven algorithm that adaptively decomposes a signal into several IMF (intrinsic mode function) components. EMD has been widely used in various industrial backgrounds [36]. In pantograph-catenary interactions [31], EMD has been proven to have good performance in identifying the structural wavelength from the contact force. Generally, EMD decomposes a given signal  into a number  of IMFs ,  and a residual  through an iterative procedure. The sum of all the IMFs and the residual precisely yields the original signal. However, the mode mixing problem is a well-known problem for traditional EMD. This problem is normally caused by signal intermittency, leading to frequency aliasing in the IMFs, which mixes disparate signal oscillations into the IMFs and impairs the physical meaning of each IMF. To resolve this problem, EEMD is proposed based on the dyadic property of EMD when dealing with white noise [37]. EEMD utilises additional white noise to ensure the full physical meaning of the IMFs.

In this section, EEMD is utilised to decompose the measured CWI, and all the IMFs and the residual are shown in Fig. 5. It is seen that the (f) through (i) IMFs show significant periodicities that contain information about the tensioning

Fig. 7. Extraction of upper envelope of CWI

section and the span length. However, extremely large peaks are observed in

Fig. 9. Generation of history from PSD: (a) history; (b) comparison of PSDs

the original signal for the first five IMFs (a)-(e). These components should be removed to eliminate the abnormal data caused by the redundant information. Through the summation of the sixth to seventeenth IMFs and the residual, the signal of the measured CWI is reconstructed as presented in Fig.

Fig. 6. Reconstructed CWI (a) with overlap; (b) without overlap

6 (a). Compared with Fig. 3 (a), it is seen that the EEMD method shows good performance in removing the abnormal points from the original data without affecting others. However, overlaps are still observed. The overlap connecting two tensioning sections may impact the pantograph head and lead to an increase in the contact force. However, the overlap does not directly contribute to the CWI. The overlaps are removed to construct the standard PSD of the CWI. The CWI without overlap is shown in Fig. 6 (b), which is only relevant to the dropper positions on the contact wire.

## Extraction of the upper envelope

As the sag between two adjacent dropper points is purely caused by gravity, the CWI is dominantly determined by the dropper position on the contact wire. Therefore, the upper envelope of the CWI is extracted because it contains all the information of the vertical position of the dropper point in the contact wire. For the catenary taken as the analysis object, the nominal interval between two adjacent droppers is 8.25 m. Therefore, a slightly smaller window of 7 m is chosen to extract the upper envelope of the CWI. As shown in Fig. 7, almost all the local peaks (dropper points) are included in the upper envelope.

## Curve fitting of CUE PSD

Employing the Yule-Walker method, the PSD of the CWI upper envelope is calculated and is presented in Fig. 8. The PSD function of track irregularity for the China high-speed network is introduced [33], and the form in Eq. (1) is adopted to fit the curve of the CUE PSD.

Fig. 8. Curve fitting of CUE PSD

Table 1. Fitting coefficients

|  | f ≤ 0.04 m-1 | 0.04 m-1 < f < 0.08 m-1 | 0.08 m-1 ≤ f |
| --- | --- | --- | --- |
| a | 6.135×10-7 | 4.494×10-15 | 2.084×10-13 |
| b | 1.54 | 7.358 | 5.84 |
| SSE | 896.8 | 1.771×10-2 | 1.071×10-5 |
| R-Square | 0.6633 | 0.979 | 0.3895 |
| AR-square | 0.662 | 0.9789 | 0.3874 |
| RMSE | 1.879 | 8.237×10-3 | 1.909×10-4 |

(1)

in which  is the CUE PSD. a and b are the fitting coefficients, which are determined by the nonlinear least square method combined with the Trust-Region algorithm. The fitting curve is shown as the red line in Fig. 8. The corresponding fitting coefficients are shown in Table 1. These coefficients are obtained with 95% confidence bounds. The statistics to evaluate the goodness of fit, namely, SSE (sum of squares due to error), R-square, AR-square (adjusted R-square) and RMSE (root mean square error), are also presented in Table 1.

The history can be generated from the PSD by the inverse Fourier transform. The frequency spectrum  is obtained by discrete sampling from the standard spectral density . The sample length is . The real and imaginary parts of  have both even and odd symmetries with respect to . Hence, the frequency spectrum  is determined by:

(2)

where  is the phase angle and obeys the uniform distribution of . Then, the history is calculated using the inverse Fourier transform as follows:

Fig. 7. Extraction of upper envelope of CWI

(3)

The generated history is presented in Fig. 9 (a). The generated PSD is compared with the original PSD in Fig. 9 (b). The good consistency shows the validity of the present method for generating the history from the PSD.

# Modelling of catenary with CWI

Fig. 11. Extraction of dropper points

In section III, a standardised PSD function for CWI is established using the measured CWI. The next task is to evaluate the effect of random CWI on the pantograph-catenary interaction performance. The flowchart of the stochastic analysis is presented in Fig. 10. In the first step, the random sample is generated using the standardised PSD function. The algorithm is illustrated in the previous section. This section presents the modelling method for a catenary with random CWI. In

Fig. 10. Flow chart of stochastic analysis of pantograph-catenary interaction performance with random CWI

section V, several numerical simulations of pantograph-catenary interaction are performed to evaluate the effect of random CWI on the contact force.

## Description of shape-finding of catenary

As the history generated from the PSD function contains all the information regarding the dropper positions on the contact wire, the dropper points are extracted by interpolation according to the spatial distribution of droppers. Fig. 11 gives an example of extracting the dropper points from the history. Then, the extracted vertical dropper positions are used to model the catenary.

To reflect CWI in the static catenary model, the shape-finding method is employed to determine the undeformed configuration of the catenary. As illustrated in Fig. 12, the deformed configuration is obtained by loading the undeformed configuration. However, in this problem, the knowns are the vertical dropper positions in the deformed configuration. The unknowns are the unstrained lengths of all the cables. Normally, this inverse problem can be solved by manual iteration [38], analytic equations [39], optimisation methodologies [25], iterative numeric procedures [40] and other complex techniques. In this paper, in combination with a nonlinear finite element approach, an iterative numeric procedure called the TCUD method is adopted to iteratively initialise the catenary.

Fig. 13. Catenary model with cable and truss elements

Fig. 12. Description of shape-finding

Nonlinear finite element approach

The catenary exhibits significant nonlinearity (including contact loss, geometrical nonlinearity and dropper slackness) under the impact of the pantograph and some other external disturbances. Therefore, the finite element approach is the most preferred method to model the catenary, compared with some other traditional methods, such as the finite difference and modal superposition methods. In this work, a nonlinear finite element approach is adopted to model a catenary with CWI. As illustrated in Fig. 13, the messenger/contact wires are modelled by the nonlinear cable element. The dropper is modelled by the truss element with a nonlinear stiffness. The steady arm is modelled by the linear truss element. The claws on the clamps of the droppers and steady arms are assumed to be lumped masses. The following gives derivations of the stiffness matrices of the cable and truss elements.

Considering a cable element with two nodes, as shown in Fig. 13, the relative distances between the two nodes can be expressed by the nodal forces as

(4a)

(4b)

(4c)

where , , and  are the relative distances along the local x-, y-, and z-axes, respectively. w is the self-weight and  is the unstrained length. E and A are Young’s modulus and the cross-sectional area, respectively. and  are the nodal forces at nodes I and J, respectively. The partial differentiation of both sides of Eq. (4) yields the incremental relationships between the relative nodal distances and nodal forces as follows:

(5)

Taking the inverse of the flexibility matrix in Eq. (5) yields the incremental equation of the cable element.

(6)

in which  is the incremental nodal force vector.  is the stiffness matrix related to the nodal displacements.  is the incremental displacement vector, and  is the stiffness matrix related to the unstrained length of the cable.

The equilibrium equation of the truss element in Fig. 13 is written as follows.

(7a)

(7b)

Fig. 14. Schematics of additional constraints

(7c)

in which  are the nodal forces at node It. ,  and  are the relative distances between two nodes along the local ,  and  axes, respectively.  is the unstrained length.  and  are Young’s modulus and the cross-sectional area, respectively. To model the slackness of the dropper,  equals zero when the dropper works in compression. Similarly, the stiffness matrix of the truss element can be obtained as:

(8)

in which  is the incremental nodal force vector,  is the stiffness matrix related to the nodal displacements, and is the incremental displacement vector.  is the stiffness matrix related to the unstrained length of the truss. It should be noted that the terms related to  and  in Eqs. (6) and (8), respectively, are only used in the TCUD method to calculate the initial shape of the catenary. These terms vanish in the dynamic simulations.

## TCUD method

The TCUD method takes the unstrained lengths  and  in Eqs. (6) and (8) as unknowns and formulates the stiffness matrix with nodal forces and unstrained lengths for each element. Assembling Eqs. (6) and (8) by FEM yields the global incremental equilibrium equation for the whole catenary as follows:

(9)

where  is the unbalanced force vector.  and  are the global stiffness matrices related to the incremental nodal displacement vector  and the incremental unstrained length vector , respectively. Assume that the total number of degrees of freedom is n and the number of elements is m. Therefore,  is an  matrix. Since the total number of unknowns  in Eq. (9) exceeds the total number of equations , Eq. (9) has infinite solutions. Hence, additional constraint conditions should be provided to suppress undesired displacement, according to the design specifications

Fig. 15. Initial configuration of catenary: (a) full geometry; (b) contact wire height

and the CWI. For the catenary in the analysis, the following additional constraint conditions are defined, as shown in Fig. 14.

- The vertical positions of the dropper point in the contact wire are restricted to describe the CWI.

- The longitudinal direction of each node is restricted to eliminate the longitudinal movement.

- The following equation is applied to the messenger and contact wires to impose traction.

(10)

The above constraint conditions can supply extra m equations to reduce the numbers of unknowns in Eq. (9). Therefore, Eq. (9) is rewritten as

(11)

where  is the unknown incremental displacement to be determined by the shape-finding, which is an (n-m)×1 vector.  is the constrained displacement vector to prevent undesired deformation of the catenary, which is an m×1 vector.  and  are the partitioned stiffness matrices corresponding to  and , respectively. Accordingly, the second term on the right-hand side of Eq. (9) vanishes, and Eq. (9) becomes

Fig. 17. Maximum contact force at normal speed: (a) Boxplot; (b) PDF

Fig. 16. Standard deviation of contact force at normal speed: (a) Boxplot; (b) PDF

(12)

In this way, the equality between the numbers of equations and unknowns ensures the unique solution of the target configuration of the catenary. The converged nodal coordinates and unstrained lengths are calculated iteratively by solving Eq. (12).

Using the dropper positions extracted from Fig. 11, the resultant static configuration of the catenary is presented in Fig. 15. It is seen from the full geometry presented in Fig. 15 (a) that the catenary model has both mid spans and overlaps. However, the CWI is only included in the mid spans. The contact wire height shown in Fig. 15 (b) shows good agreement with the distribution of the dropper points extracted from Fig. 11, which demonstrates the validity of the present shape-finding method.

# Stochastic analysis with CWI

The present modelling method for the catenary has been validated against European standard [41], benchmark [42] and measured [43] and [44] data. Using the updated catenary model with random CWI and a lumped mass model for the pantograph, stochastic analysis is performed to reveal the effect of random CWI on the contact force, which is of great importance for the current collection quality. According to the technical criteria for the pantograph-catenary interaction [45], the contact force standard deviation , which is filtered within 0-20 Hz, is the most important index indicating the fluctuation of the contact force with respect to its mean value. The maximum contact

Fig. 20. Standard deviation of contact force with speed upgrade: (a) Boxplot; (b) PDF

Fig. 18. Statistical maximum contact force at normal speed: (a) Boxplot; (b) PDF

Fig. 19. Statistical minimum contact force at normal speed: (a) Boxplot; (b) PDF

force  is strictly limited to prevent damage to the catenary from the pantograph. To facilitate the comparison in the following analysis, the mean contact force in each simulation is tuned to fulfil the maximum , according to the standard [45], namely:

(13)

where v is the train speed. Normally, the statistical maximum and minimum contact forces and  are also important indicators for evaluating the dynamic performance of the pantograph-catenary system. These parameters are defined as follows:

Fig. 22. Statistical maximum contact force with speed upgrade: (a) Boxplot; (b) PDF

Fig. 21. Maximum contact force with speed upgrade: (a) Boxplot; (b) PDF

(14a)

(14b)

Based on the idea of the Monte Carlo method, a total of 500 numerical simulations are performed to describe the stochastics of the CWI at each speed. The results with normal speed v=300 km/h are analysed first. Then, the results with speed upgrades are evaluated.

## Statistical analysis with normal speed

The contact force statistics of 500 simulations are displayed in two ways. The first is the boxplot, which illustrates the dispersion and fluctuation of the results. The other is the PDF, which describes the probability of the appearance of each value. To facilitate the comparison, the ideal results without CWI are denoted by red dashed-dotted lines in each figure. The results for the contact force standard deviation  are shown in Fig. 16. The ideal  without CWI is 31.13 N. The presence of CWI causes the dispersion of . The largest  reaches 35.15 N, and the smallest value is only 30.33 N, which is even better than the ideal value. It is also seen that 87% of the resultant  with CWI are worse than the ideal case, while 14% of the results are better.

The results for the maximum contact force  are shown in Fig. 17. When the CWI is presented, 93% of resultant  are worse than the ideal case. The largest  reaches 266.06 N, which is 14% larger than the ideal value. The statistical maximum contact force  is shown in Fig. 18. When the CWI is presented, 87% of the resultant  are larger than the ideal value. The largest  reaches 262.41 N, which is 5% larger than the ideal value. The statistical minimum contact force  is shown in Fig. 19. Similar to , 87% of resultant  with CWI are worse than the ideal case. The smallest

Fig. 23. Statistical minimum contact force with speed upgrade: (a) Boxplot; (b) PDF

is 52.2 N, which is 22% smaller than the ideal value.

## Statistical analysis with speed upgrade

To analyse the effect of random CWI on the pantograph-catenary interaction at higher operating speeds, a number of simulations are performed for three different speeds: 330 km/h, 350 km/h and 370 km/h. Similar to the above analysis, the boxplot and the PDF are utilised to display the statistics. The dashed-dotted lines denote the ideal results without CWI. The results for , ,  and  are shown in Figs. 20-23, respectively. At each speed, the random CWI causes significant dispersions of , ,  and . Over 70% of the resultant ,  and  with CWI are larger than the ideal values at the corresponding speeds. Accordingly, over 70% of the resultant  with CWI are smaller than the ideal values at the corresponding speeds. Particularly at 370 km/h, 1.6% of the resultant  and 51% of the resultant  exceed the safety criteria of 350 N, which may damage the catenary and cause severe accidents. In the current standard, the random effect of CWI is not included in the assessment of the current collection quality, which may result in up to 20% error in the contact force statistics with respect to a realistic value.

# Conclusions

The contact wire irregularity (CWI) portrays a realistic and common disturbance to the stable operation of a pantograph-catenary system. In this paper, a procedure is proposed to construct the standardised PSD function for the CWI based on measurement data from the China high-speed network. Through the frequency analysis of realistic CWI, it is found that CWI is dominantly determined by the vertical position of each dropper point on the contact wire. The sag between two adjacent droppers is purely caused by gravity. The upper envelope of the CWI is extracted to describe the vertical dropper points in the contact wire. The form of the standardised PSD for the CWI upper envelope is constructed. Then, employing the TCUD method, a catenary model with the real CWI considered is established, in which the vertical position of the dropper and steady arm in the contact wire are correctly placed according to history generated from the PSD. The effect of the random CWI on the pantograph-catenary interaction is studied through a number of numerical simulations. The results indicate that CWI is responsible for greater dispersion in the contact force statistics, including the standard deviation, maximum value, statistical maximum and minimum values. Mostly, CWI has a negative effect on the contact quality of a pantograph-catenary system. Especially at 370 km/h, CWI causes the maximum contact force to exceed the safety criteria. It is suggested that the CWI should be included in the numerical tools to assess the pantograph-catenary interaction performance.

It should be pointed out that the CWI analysed in this paper is geometric irregularity caused by structural distortion, which has a long wavelength and directly affects the low-frequency interaction performance. However, another type of CWI is the short-wavelength irregularity, which is mainly caused by contact wire wear, local cants, and defective claws or clamps. This type of CWI may affect the high-frequency interaction performance of a pantograph-catenary and cause a local increase in contact force. More measurement data for the short-wavelength CWI will be collected and analysed in the future.

References

[1]	J. Zhang, W. Liu, and Z. Zhang, “Study on Characteristics Location of Pantograph-Catenary Contact Force Signal Based on Wavelet Transform,” IEEE Trans. Instrum. Meas., vol. 68, no. 2, pp. 402–411, 2019.

[2]	Y. Song, Z. Liu, F. Duan, X. Lu, and H. Wang, “Study on Wind-Induced Vibration Behavior of Railway Catenary in Spatial Stochastic Wind Field Based on Nonlinear Finite Element Procedure,” J. Vib. Acoust., vol. 140, no. 1, p. 011010, 2017.

[3]	Z. Wang, Y. Song, Z. Yin, R. Wang, and W. Zhang, “Random response analysis of axle-box bearing of a high-speed train excited by crosswinds and track irregularities,” IEEE Trans. Veh. Technol., 2019.

[4]	Y. Chen et al., “A temperature compensated fibre Bragg grating (FBG)-based sensor system for condition monitoring of electrified railway pantograph,” 25th Int. Conf. Opt. Fiber Sensors, vol. 10323, p. 103236T, 2017.

[5]	S. Ostlund, A. Gustafsson, L. Buhrkall, and M. Skoglund, “Condition monitoring of pantograph contact strip,” pp. 37–37, 2009.

[6]	S. Midya, D. Bormann, T. Schütte, and R. Thottappillil, “DC component from pantograph arcing in AC traction system-influencing parameters, impact, and mitigation techniques,” IEEE Trans. Electromagn. Compat., vol. 53, no. 1, pp. 18–27, 2011.

[7]	D. Hai He, R. R. Manory, and N. Grady, “Wear of railway contact wires against current collector materials,” Wear, vol. 215, no. 1–2, pp. 146–155, 1998.

[8]	W. Liu et al., “Multi-Objective Performance Evaluation of the Detection of Catenary Support Components Using DCNNs,” IFAC-PapersOnLine, vol. 51, no. 9, pp. 98–105, 2018.

[9]	H. Wang, Z. Liu, A. Nunez, and R. Dollevoet, “Entropy-Based Local Irregularity Detection for High-Speed Railway Catenaries With Frequent Inspections,” IEEE Trans. Instrum. Meas., pp. 1–12, 2018.

[10]	Y. Song, Z. Liu, H. Wang, X. Lu, and J. Zhang, “Nonlinear analysis of wind-induced vibration of high-speed railway catenary and its influence on pantograph–catenary interaction,” Veh. Syst. Dyn., vol. 54, no. 6, pp. 723–747, 2016.

[11]	Y. Song, Z. Liu, H. Wang, J. Zhang, X. Lu, and F. Duan, “Analysis of the galloping behaviour of an electrified railway overhead contact line using the non-linear finite element method,” Proc. Inst. Mech. Eng. Part F J. Rail Rapid Transit, vol. 232, no. 10, pp. 2339–2352, 2018.

[12]	A. Carnicero, J. R. Jimenez-Octavio, C. Sanchez-Rebollo, A. Ramos, and M. Such, “Influence of Track Irregularities in the Catenary-Pantograph Dynamic Interaction,” J. Comput. Nonlinear Dyn., vol. 7, no. 4, p. 041015, Jul. 2012.

[13]	J. Pombo and J. Ambrosio, “Environmental and track perturbations on multiple pantograph interaction with catenaries in high-speed trains,” Comput. Struct., vol. 124, pp. 88–101, 2013.

[14]	P. Boffi et al., “Optical fiber sensors to measure collector performance in the pantograph-catenary interaction,” in IEEE Sensors Journal, 2009, vol. 9, no. 6, pp. 635–640.

[15]	G. Gao, J. Hao, W. Wei, H. Hu, G. Zhu, and G. Wu, “Dynamics of Pantograph-Catenary Arc during the Pantograph Lowering Process,” IEEE Trans. Plasma Sci., vol. 44, no. 11, pp. 2715–2723, 2016.

[16]	S. Midya, D. Bormann, Z. Mazloom, T. Schütte, and R. Thottappillil, “Conducted and radiated emission from pantograph arcing in AC traction system,” 2009 IEEE Power Energy Soc. Gen. Meet. PES ’09, pp. 1–8, 2009.

[17]	E. Di Stefano, C. A. Avizzano, M. Bergamasco, P. Masini, M. Menci, and D. Russo, “Automatic inspection of railway carbon strips based on multi-modal visual information,” IEEE/ASME Int. Conf. Adv. Intell. Mechatronics, AIM, pp. 178–184, 2017.

[18]	G. Bucca and A. Collina, “A procedure for the wear prediction of collector strip and contact wire in pantograph-catenary system,” Wear, vol. 266, no. 1–2, pp. 46–59, 2009.

[19]	H. Wang, A. Núñez, Z. Liu, Y. Song, F. Duan, and R. Dollevoet, “Analysis of the evolvement of contact wire wear irregularity in railway catenary based on historical data,” Veh. Syst. Dyn., vol. 56, no. 8, pp. 1207–1232, 2018.

[20]	C. J. Cho and H. Ko, “Video-based dynamic stagger measurement of railway overhead power lines using rotation-invariant feature matching,” IEEE Trans. Intell. Transp. Syst., vol. 16, no. 3, pp. 1294–1304, 2015.

[21]	J. Chen, Z. Liu, H. Wang, A. Nunez, and Z. Han, “Automatic defect detection of fasteners on the catenary support device using deep convolutional neural network,” IEEE Trans. Instrum. Meas., vol. 67, no. 2, pp. 257–269, 2018.

[22]	X. Wu, P. Yuan, Q. Peng, C. W. Ngo, and J. Y. He, “Detection of bird nests in overhead catenary system images for high-speed rail,” Pattern Recognit., vol. 51, pp. 242–254, 2016.

[23]	Y. Han, Z. Liu, D. J. Lee, W. Liu, J. Chen, and Z. Han, “Computer vision–based automatic rod-insulator defect detection in high-speed railway catenary system,” Int. J. Adv. Robot. Syst., vol. 15, no. 3, 2018.

[24]	P. Nåvik, A. Rønnquist, and S. Stichel, “The use of dynamic response to evaluate and improve the optimization of existing soft railway catenary systems for higher speeds,” Proc. Inst. Mech. Eng. Part F J. Rail Rapid Transit, vol. 230, no. 4, pp. 1388–1396, 2015.

[25]	P. Antunes, J. Ambrósio, and J. Pombo, “Catenary Finite Element Model Initialization using Optimization,” in Proceedings of the Third International Conference on Railway Technology: Research, Development and Maintenance, 2016, vol. 110.

[26]	P. Nåvik, A. Rønnquist, and S. Stichel, “Variation in predicting pantograph–catenary interaction contact forces, numerical simulations and field measurements,” Veh. Syst. Dyn., vol. 55, no. 9, pp. 1265–1282, 2017.

[27]	W. Zhang, G. Mei, and J. Zeng, “A study of pantograph/catenary system dynamics with influence of presag and irregularity of contact wire,” Veh. Syst. Dyn., vol. 37, no. SUPPL., pp. 593–604, 2003.

[28]	Y. H. Cho, K. Lee, Y. Park, B. Kang, and K. N. Kim, “Influence of contact wire pre-sag on the dynamics of pantographrailway catenary,” Int. J. Mech. Sci., vol. 52, no. 11, pp. 1471–1490, 2010.

[29]	O. Vo Van, J. P. Massat, and E. Balmes, “Waves, modes and properties with a major impact on dynamic pantograph-catenary interaction,” J. Sound Vib., vol. 402, pp. 51–69, 2017.

[30]	H. Wang et al., “Detection of Contact Wire Irregularities Using a Quadratic Time-Frequency Representation of the Pantograph-Catenary Contact Force,” IEEE Trans. Instrum. Meas., vol. 65, no. 6, pp. 1385–1397, 2016.

[31]	Z. Liu, H. Wang, R. Dollevoet, Y. Song, A. Nunez, and J. Zhang, “Ensemble EMD-Based Automatic Extraction of the Catenary Structure Wavelength from the Pantograph-Catenary CEnsemble EMD-Based Automatic Extraction of the Catenary Structure Wavelength from the Pantograph-Catenary Contact Forceontact Force,” IEEE Trans. Instrum. Meas., vol. 65, no. 10, pp. 2272–2283, 2016.

[32]	A. Collina, F. Fossati, M. Papi, and F. Resta, “Impact of overhead line irregularity on current collection and diagnostics based on the measurement of pantograph dynamics,” Proc. Inst. Mech. Eng. Part F J. Rail Rapid Transit, vol. 221, no. 4, pp. 547–559, 2007.

[33]	“TBT 3352-2014 PSD of ballastless track irregularities of high-speed railway. 2015.” .

[34]	M. J. Goodwin, “Dynamics of railway vehicle systems,” J. Mech. Work. Technol., 1987.

[35]	L. Frýba, Dynamics of Railway Bridges. Telford, 1996.

[36]	Y. Lei, J. Lin, Z. He, and M. J. Zuo, “A review on empirical mode decomposition in fault diagnosis of rotating machinery,” Mechanical Systems and Signal Processing. 2013.

[37]	Z. Wu and N. E. Huang, “Ensemble empirical mode decomposition: A noise-assisted data analysis method,” Adv. Adapt. Data Anal., 2009.

[38]	O. Lopez-Garcia, A. Carnicero, and V. Torres, “Computation of the initial equilibrium of railway overheads based on the catenary equation,” Eng. Struct., vol. 28, no. 10, pp. 1387–1394, 2006.

[39]	M. Tur, E. García, L. Baeza, and F. J. Fuenmayor, “A 3D absolute nodal coordinate finite element model to compute the initial configuration of a railway catenary,” Eng. Struct., vol. 71, pp. 234–243, 2014.

[40]	Y. Song, Z. Liu, H. Ouyang, H. Wang, and X. Lu, “Sliding Mode Control with PD Sliding Surface for High-Speed Railway Pantograph-Catenary Contact Force under Strong Stochastic Wind Field,” Shock Vib., vol. 2017, pp. 1–16, Jan. 2017.

[41]	EN 50318. Railway applications - Current collection systems - Validation of simulation of the dynamic interaction between pantograph and overhead contact line. Brussels: European Standards (EN), 2018.

[42]	S. Bruni et al., “The results of the pantograph-catenary interaction benchmark,” Veh. Syst. Dyn., vol. 53, no. 3, pp. 412–435, 2015.

[43]	Y. Song, Z. Liu, H. Wang, X. Lu, and J. Zhang, “Nonlinear modelling of high-speed catenary based on analytical expressions of cable and truss elements,” Veh. Syst. Dyn., vol. 53, no. 10, pp. 1455–1479, 2015.

[44]	Y. Song, Z. Liu, F. Duan, Z. Xu, and X. Lu, “Wave propagation analysis in high-speed railway catenary system subjected to a moving pantograph,” Appl. Math. Model., vol. 59, pp. 20–38, 2018.

[45]	EN 50367. Railway applications — Current collection systems — Technical criteria for the interaction between pantograph and overhead line., no. August 2013. 2016.

Yang Song (S’16–M’19) received a Ph.D. degree in electrical engineering from Southwest Jiaotong University, Sichuan China, in 2018. He worked as a Research Fellow with the Institute of Railway Research, School of Computing and Engineering, University of Huddersfield, UK from 10/2018 to 09/2019. He is currently a postdoctoral researcher in the Department of Structural Engineering, Norwegian University of Science and Technology, Norway. His research interests involve the assessment of railway pantograph-catenary interactions, the wind-induced vibration of long-span structures in railway transportation, and coupling dynamics in railway engineering.

Zhigang Liu (M’06–SM’16) received a Ph.D. degree in power systems and their automation from Southwest Jiaotong University, Sichuan, China, in 2003. He is currently a full professor in the School of Electrical Engineering, Southwest Jiaotong University, China. His current research interests include the vehicle-grid electrical relationships in high-speed railways, pantograph-catenary interactions, and status assessments. Prof. Liu was awarded the fellowship of the Institute of Engineering and Technology in 2017. He is an associate editor for IEEE Transactions on Instrumentation and Measurement, IEEE Transactions on Vehicular Technology and IEEE Access.

Anders Rønnquist received a Ph.D. degree from Norwegian University of Science and Technology, Trondheim, Norway, in 2005. He is currently a full professor in the Department of Structural Engineering, Norwegian University of Science and Technology. His research interests include structural dynamics, wind engineering, structural monitoring and system identification for electric railways.

Petter Nåvik received a Ph.D. degree from Norwegian University of Science and Technology, Trondheim, Norway, in 2016. He is currently employed as a Postdoctoral Researcher in the structural dynamics group at the Department of Structural Engineering, Norwegian University of Science and Technology, NTNU. His main task is to investigate the structural dynamics of railway catenary systems and pantograph-catenary interactions. His research activities include both numerical analyses and field studies. His research interests include structural dynamics, the dynamics of railway catenary systems, structural monitoring, operational modal analysis, and finite element modelling.

Zhendong Liu received a Ph.D. degree from KTH Royal Institute of Technology, Stockholm, Sweden in 2017. He is currently a postdoctoral researcher at KTH Royal Institute of Technology, Stockholm, Sweden. His research interests include pantograph-catenary dynamics and reduced energy usage at train at train operation.

This work was supported in part by the National Natural Science Foundation of China (U1734202; 51977182).

Yang Song, Anders Rønnquist and Petter Nåvik are with the Department of Structural Engineering, Norwegian University of Science and Technology, Trondheim, 7491, Norway. (e-mail: y.song_ac@hotmail.com, anders.ronnquist@ntnu.no and petter.r.navik@ntnu.no).

Zhigang Liu is with the Electrical Engineering Department, Southwest Jiaotong University, Chengdu, 610031 China. (e-mail: liuzg_cd@126.com).

Zhendong Liu is with the Department of Aeronautical and Vehicle Engineering, KTH Royal Institute of Technology, Teknikringen 8, 10044 Stockholm, Sweden. (e-mail: zhendong@kth.se)